# Flow Modifier

Ross Wickham, NWW

June 2019

## Mod_puls_config

Example:

| tblName | dssPath | nSubreaches |
| --- | --- | --- |
| peck_to_pot_cf | //PECK TO POTLACH_CF/STORAGE-FLOW///NWW_CLEARWATER/ | 24 |

‘tblName’ defines the name to be used in the ‘flow_eqn_config’ function definition. E.g., the function string would be:

modifiedPulsRouting peck_to_pot_cf

You also need to define the DSS path and the number of subreaches.  This process assumes the Muskingum X value is zero, but it wouldn’t be too difficult to change.  The DSS path MUST have a C part of ‘STORAGE-FLOW’, or the computation will not work

# Tips

If you’re not re-producing the full time period (e.g., ESP data are available Aug-Jan, but the local you’re computing only goes from Aug-Oct), then the routing is most likely going unstable.  Check that there aren’t any very small reaches (<1 mile) that could be removed from the routing, and reasonably ignored.

Apply any multiplication factors after routing to avoid instabilities.

# Example